Emtron Technologies  
Flat No-101, B3 Wing, 1st Floor, Divyam Hights,     Gilbert Hill, Shreenath Nagar, Andheri –West,           Mumbai-58 
+91-8080181911                                                                    E-mail: emtron.tech@gmail.com, www.emtrontech.in

ARM Processors

Development board

Lab Manual

Introduction:

The LPC2141/2/4/6/8 microcontrollers are based on a 32/16 bit ARM7TDMI-S CPU with real-time emulation and embedded trace support, that combines the microcontroller with embedded high speed flash memory ranging from 32 kB to 512 kB. A 128-bit wide memory interface and a unique accelerator architecture enable 32-bit code execution at the maximum clock rate. For critical code size applications, the alternative 16-bit Thumb mode reduces code by more than 30 % with minimal performance penalty.

Due to their tiny size and low power consumption, LPC2141/2/4/6/8 are ideal for applications where miniaturization is a key requirement, such as access control and point-of-sale. A blend of serial communications interfaces ranging from a USB 2.0 Full Speed device, multiple UARTS, SPI, SSP to I2Cs and on-chip SRAM of 8 kB up to 40 kB, make these devices very well suited for communication gateways and protocol converters, soft modems, voice recognition and low end imaging, providing both large buffer size and high processing power. Various 32-bit timers, single or dual 10-bit ADC(s), 10-bit DAC, PWM channels and 45 fast GPIO lines with up to nine edge or level sensitive external interrupt pins make these microcontrollers particularly suitable for industrial control and medical systems.

LPC2148 Features:

• 16/32-bit ARM7TDMI-S microcontroller in a tiny LQFP64 package.

• 8 to 40 kB of on-chip static RAM and 32 to 512 kB of on-chip flash program memory. 128 bit wide interface/accelerator enables high speed 60 MHz operation.

• In-System/In-Application Programming (ISP/IAP) via on-chip boot-loader software.

Single flash sector or full chip erase in 400 ms and programming of 256 bytes in 1 ms.

• EmbeddedICE RT and Embedded Trace interfaces offer real-time debugging with the on-chip RealMonitor software and high speed tracing of instruction execution.

• USB 2.0 Full Speed compliant Device Controller with 2 kB of endpoint RAM. In addition, the LPC2146/8 provide 8 kB of on-chip RAM accessible to USB by DMA.

• One or two (LPC2141/2 vs. LPC2144/6/8) 10-bit A/D converters provide a total of 6/14 analog inputs, with conversion times as low as 2.44 μs per channel.

• Single 10-bit D/A converter provides variable analog output.

• Two 32-bit timers/external event counters (with four capture and four compare channels each), PWM unit (six outputs) and watchdog.

• Low power real-time clock with independent power and dedicated 32 kHz clock input.

• Multiple serial interfaces including two UARTs (16C550), two Fast I2C-bus (400 kbit/s), SPI and SSP with buffering and variable data length capabilities.

• Vectored interrupt controller with configurable priorities and vector addresses.

• Up to 45 of 5 V tolerant fast general purpose I/O pins in a tiny LQFP64 package.

• Up to nine edge or level sensitive external interrupt pins available. 60 MHz maximum CPU clock available from programmable on-chip PLL with settling time of 100 μs.

• On-chip integrated oscillator operates with an external crystal in range from 1 MHz to 30 MHz and with an external oscillator up to 50 MHz.

• Power saving modes include Idle and Power-down.

• Individual enable/disable of peripheral functions as well as peripheral clock scaling for additional power optimization.

• Processor wake-up from Power-down mode via external interrupt, USB, Brown-Out Detect (BOD) or Real-Time Clock (RTC).

• Single power supply chip with Power-On Reset (POR) and BOD circuits:

– CPU operating voltage range of 3.0 V to 3.6 V (3.3 V ± 10 %) with 5 V tolerant I/O pads.

Board Features:

This ARM board is an evaluation board using LPC2148 ARM7TMDI based microcontroller. The LPC2148 microcontroller has 512KB of internal flash and 32+8K RAM. Following are the salient features of the board.

Power supply: DC 9V power with LED link.

On-board linear regulators generate +3.3V/500mA and +5v/500mA from power supply.

USB connector (as alternate power source).

RS232 connectors (2)

JTAG connector

SD/MMC connector

ZigBee connector

USB B-type connector with Link-LED

2 line X 16 character LCD with back light control

Configurable for manual and automatic program download (ISP) via serial port.

Controllable LED on Port P0.31

Connectors:

I/O1:	 	2 line X 16 character LCD

VBT:	 	3.3V battery connector

JTAG:	 	JTAG connector

6x1 PINS: 	GPIO for MOTOR & KBD Interface

ISP: 		In System Programming

RESET: 	Reset SW of ARM-7

INT-0: 	 INT-0 PORT PIN

ADC-0: 	ADC 0 PORT PIN

WiFi:	 	ESP8266WiFi Module Interface

Z1: 		Bluetooth Interface

DAC: 		AOUT

X1: 		USB Image I/P

X2: 		Mini USB Program

SUP: 		9V/12V DC supply

COM:	 	UART1

COM2: 	UART2

Q2: 		12 MHz Crystal

SDRAM: 	SPI Port

5x2 BOX: GPIO PINS (ADC PORT, JTAG PORT)

System Requirements

Windows XP

Serial port

USB port

Connecting the hardware:

After unpacking the board connect a DC supply of 9V/800mA to the DC jack to power the board. The board can also be powered through USB. To test all the features on the board you would need the following accessories:

- USB cable

Once you have all these accessories connected to the board you can run through a simple test to verify that all the peripherals are working fine. Please refer to the ‘Hardware Configuration’ section for testing all the peripherals.

Programming the board:

This board can be programmed through  20-pin  JTAG or through serial port using ‘Flash Magic’. ‘Flash Magic’ is a freeware windows utility used download the hex file format onto the board. Flash Magic can be downloaded from here http://www.flashmagictool.com/. If your PC does not have a serial port; use a USB to serial converter to download the hex file using the Flash Magic utility. For programming with JTAG your system should have a USB port and the supporting IDE which can communicate to the processor core over JTAG interface. We have successfully tested  board with uLink JTAG IDE. A LINUX utility to download the hex file can be found here http://www.pjrc.com/arm/lpc2k_pgm/.

Programming board Through ISP:

The board can be programmed through ISP in two modes:

- Auto Mode

- Manual Mode

Auto Mode:

To program in Auto mode you need a full serial cable. Short the jumper pins 1 & 2 of  ISP and connect Mini USB cable . When board is powered ON black boxes will be displayed on LCD. Open Flash Magic tool, select the appropriate COM port (Verify In Windows -> Control Panel-> Device Manager-> Ports-> COM Port Number->), set the Baud rate to less than or equal to 38400 bps, select device as LPC2148, interface as 'None (ISP)' and oscillator frequency as 12MHz. Specify the path of your HEX file and click START. The status is shown at the bottom on the Flash Magic window. In the 'Step 4 - Options' check 'Verify after programming' and 'Fill unused flash' options. Checking the 'Set Code Read Prot' option will not allow you will program with JTAG. So keep it unchecked unless required.

Note:

1. In Auto mode under the 'Options' tab select 'Advanced options'. In this under 'Hardware Config' tab make sure the options 'Use DTR and RTS to control RTS and P0.14' and 'Keep RTS asserted while COM port open' are checked. The values of T1 and T2 are set to 100ms and 200ms by default.

2. After programming the board in Auto mode you should disconnect the serial cable and remove the Jumper and reset the controller. This is a known issue.

2. Manual Mode:

To program in Manual mode : and connect the USB cable to PC. To make the board enter programming mode:

- Verify in Windows -> Control Panel-> Device Manager-> Ports-> COM Port Number-> (If not detected install FT232 Driver )

- I- ON ISP ( Switch ON to make it ISP Mode)

- RESET ARM-7 Board  (the controller enters the boot loader mode if during RESET the ISP pin is low)

- Flash Magic Settings:

- Select Device: LPC2148

- COM Port: COMxx (As per Device Manager Indication)

- Baud Rate: 9600

- Interface: (None) ISP

- Oscillator: 12 Mhz

- Erase Blocks used by Hex file: yes

- Advanced Options: Use DTR and RTS to control RST and ISP pin: NO

- Switch OFF ISP Switch

- Press REST Switch to execute the program

COM1 & COM2:

Open the hyper terminal on your PC and write USRT programs to test com port.

To test the UART you can use either a full modem or half modem cable.

JTAG connector:

To enable debugging on the board connect jumper to DBG and connect the JTAG to debug port. We have successfully tested the board with JTAG interface using uLink  JTAG. To test this feature you need to have the necessary software support on your PC.

User Interface Switch:

The three pin connector OPTO-1 (P0.16), OPTO-2 (P1.25) is connected to one of the external interrupt lines of LPC. To test this interface simply press the switch and you should hear the beep sound on the buzzer. This confirms that both the interrupt line and the buzzer module are working fine. Please ensure that you have connected the buzzer jumper appropriately.

Buzzer:

Connect external buzzer to two pin connector (DAC), when the board is turned on or RESET you will hear a beep after few seconds. This is how the user can confirm the status of the Buzzer.

SD/MMC connector:

Insert a SD card in the SD card holder; the status of the SD card will be displayed on LCD upon power cycle or reset of the board. If the SD card is inserted properly “SD card – OK” is displayed on LCD else it displays “SD card- Not OK”. During manufacturing the board is tested with Kingston’s 1GB SD card.

Note: The SD/MMC card being tested should be formatted with FAT file system (Not FAT32 or NTFS format).

LCD display:

To enable the LCD connect all 16 pins of I/O1 to LCD. A default message “Emtron Technologies” will be displayed and later status of SD/MMC and I2C is displayed.  An optional 20-pin I/O2 port is provided to interface 64x128 graphics LCD.

ZigBee/Bluetooth:

For wireless transmission between two ARM kit, the ZigBee protocol can used. ZigBee is a wireless technology developed as an open global standard to address the unique needs of low-cost, low-power wireless M2M networks. The ZigBee standard operates on the IEEE 802.15.4 physical radio specification and operates in unlicensed bands including 2.4 GHz, 900 MHz and 868 MHz. This port is available on Z connector of the board.

RTC :

A SPI connector SDCARD is provided for external RTC. Internal ARM RTC is connected with 32.767KHz crystal.

ADC :

The ADC port is provided on VADC, IADC connectors. To test the ADC interface connect external potentiometer and rotate the POT, as the POT position varies the output number on LCD varies.

Overview:

For the working with board there are certain tools that need to be installed. The tools required to work with bard are:

•	Flash Magic.

The flash magic tool can be downloaded from the following link:

http://www.flashmagictool.com/

For LINUX machines you may use http://www.pjrc.com/arm/lpc2k_pgm/

•	Tool chain:

To be able to generate the hex or the binary file the user needs to install the tool chain for ARM based microcontrollers. Any toolchain can be used as long as it is able to generate the necessary files for downloading onto the board. Here are few toolchain suggestions:

- GNUARM ToolChain: http://winarm.scienceprog.com/winarm-tools/prepare-gnuarm-compiler-toolchain-for-windows.html

- WINARM : Rowley Crossworks IDE: http://www.rowley.co.uk/arm/

Board Circuit diagram:

Board Layout:

Board Part list:

Part     Value          Device         Package                   		Library

C1       0.1            C-EUC1206      C1206                     		resistor

C2       0.1            C-EUC1206      C1206                     		resistor

C3       0.1            C-EUC1206      C1206                     		resistor

C4       0.1            C-EUC1206      C1206                     		resistor

C5       104            C-EUC1206      C1206                     	resistor

C6       22p            C-EUC1206      C1206                     	resistor

C7       22p            C-EUC1206      C1206                     	resistor

C8       22p            C-EUC1206      C1206                     	resistor

C9       22p            C-EUC1206      C1206                     	resistor

C10      104            C-EUC1206      C1206                    	resistor

C11      104            C-EUC1206      C1206                    	resistor

C12      104            C-EUC1206      C1206                    	resistor

C13      104            C-EUC1206      C1206                    	resistor

C14      10u            CPOL-EUB45181A                        	resistor

C15      10u            CPOL-EUB45181A                        	resistor

C16      0.1            C-EUC1206      C1206                     	resistor

C17      18p            C-EUC1206      C1206                    	resistor

C18      18p            C-EUC1206      C1206                    	resistor

C19      0.1            C-EUC1206      C1206                     	resistor

C20      0.1            C-EUC1206      C1206                     	resistor

C21      0.1            C-EUC1206      C1206                     	resistor

COM1                    F09HP          F09HP                     		con-subd

COM2                    PINHD-1X3      1X03                      	pinhead

D1       4448           1N4148DO35-7   DO35-7                    	diode

DAC                     PINHD-1X2      1X02                        	pinhead

DBG                     PINHD-1X2      1X02                        	pinhead

I/O1                    PINHD-1X16     1X16                         	pinhead

I/O2                    PINHD-1X20     1X20                         	pinhead

IADC                    PINHD-1X5      1X05                        	pinhead

IC1      LPC2148FBD64   LPC2148FBD64            		TSQFP-64N

IC2     78XXS          78XXS                                              	v-reg

IC3      LD117ASTR      LD117ASTR      SOT223         	v-reg

IC4      MAX3243UI      MAX3243UI      TSSOP28        	maxim

IO                      PINHD-1X5      1X05                              	pinhead

ISP                     PINHD-1X2      1X02                             	pinhead

JTAG                PINHD-2X10     2X10                             	pinhead

LED1                    LEDSMT1206     1206                        	led

LED2                    LEDSMT1206     1206                        	led

OK1      PC817          PC817          DIL04                        	optocoupler

OPTO-1                  PINHD-1X3      1X03                        	pinhead

OPTO-2                  PINHD-1X3      1X03                      	pinhead

Q1       32.768K     CRYSTALTC26H   TC26H            	crystal

Q2       12M            CRYSTALHC49U70 HC49U70    	crystal

R1       10k            R-EU_R1206     R1206                     	resistor

R2       10k            R-EU_R1206     R1206                     	resistor

R3       10k            R-EU_R1206     R1206                     	resistor

R4       10k            R-EU_R1206     R1206                     	resistor

R5       10k            R-EU_R1206     R1206                     	resistor

R6       10k            R-EU_R1206     R1206                     	resistor

R7       10k            R-EU_R1206     R1206                     	resistor

R8       10k           R-EU_R1206     R1206                     	resistor

R9       1k             R-EU_R1206     R1206                    		resistor

R10      100k        R-EU_R1206     R1206                     	resistor

R11      10k           R-EU_R1206     R1206                    	resistor

R12      2.2k          R-EU_R1206     R1206                    	resistor

R13      33             R-EU_R1206     R1206                     	resistor

R14      33             R-EU_R1206     R1206                     	resistor

R15      47k           R-EU_R1206     R1206                     	resistor

R16      1k             R-EU_R1206     R1206                     	resistor

R17      10k           R-EU_R1206     R1206                     	resistor

R18      1k             R-EU_R1206     R1206                     	resistor

R19      1.5k           R-EU_R1206     R1206                     	resistor

R20      56E            R-EU_R1206     R1206                     	resistor

R21                     R-TRIMM64W     RTRIM64W           	resistor

R22      1k             R-EU_R1206     R1206                       	resistor

R23      100e           R-EU_R1206     R1206                     	resistor

RL                            PINHD-1X2      1X02                      	pinhead

RN1                          G08R           SIL9                            	resistor-sil

RN2                         G06R           SIL7                             	resistor-sil

S1                             10-XX          B3F-10XX                   	switch-omron

SDRAM                   PINHD-1X6      1X06                      	pinhead

SUP                          PINHD-1X2      1X02                      	pinhead

U$1   SDCARDVAR2     SDCMF-10915W010              	sdcmf10915

VADC                    PINHD-1X5      1X05                         	pinhead

VBAT                    PINHD-1X2      1X02                         	pinhead

X1                          PN61729-S      		                    	con-berg

Z                            PINHD-1X6      1X06                	        	pinhead

Experiment List of ARM-7 (LPC2148)

Refer LPC2148 Examples Codes Folder

Experiment-1: (code: 1_Blinky) LED connected on (Port P0.31) of LPC2148

Experiment-2: (code: 1_Blinky_Input) LED connected on (Port P0.31) of LPC2148, Switch on PORT0.16

Experiment-3: (code: 2_Test_LCD) 16x2 LCD, 8-bit Mode, RS=PORT0.2, RW=PORT0.3, E=PORT0.4, D0 to D7 = PORT1.16 to PORT1.23

Experiment-4: (code: 3_ADC_LCD) 16x2 LCD, 8-bit Mode, RS=PORT0.2, RW=PORT0.3, E=PORT0.4, D0 to D7 = PORT1.16 to PORT1.23 & ADC Channel NO-0 (PORT0.28)

Experiment-4: (code: 4_DAC) DAC sine waveform generation and amplification (PORT0.25)

Experiment-5: (code: 5_ADC_DAC_LCD) 16x2 LCD, 8-bit Mode, RS=PORT0.2, RW=PORT0.3, E=PORT0.4, D0 to D7 = PORT1.16 to PORT1.23 & ADC Channel NO-0 (PORT0.28) & sine waveform generation and amplification (PORT0.25)

Experiment-6: (code: 6_Serial)  Serially communicate String “Hello World” to Computer Serial Terminal using 9600 baud rate, 8-bits, No parity, 1-Stop bit. Use UART-0 of the LPC2148 ARM Processor is working on 60MHz.

Experiment-7: (code: 7_RTC) Initialize Internal RTC of LPC2148 for Seconds, Minutes & Hours and Display the data on LCD of  16x2 LCD, 8-bit Mode, RS=PORT0.2, RW=PORT0.3, E=PORT0.4, D0 to D7 = PORT1.16 to PORT1.23

Experiment-9: (code: 8_Serial_ADC) read ADC data of Channel No-0 of LPC2148 and transmit serially through UART-0 to Computer Serial Terminal using 9600 baud rate, 8-bits, No parity, and 1-Stop bit.

Project-1: (code: 9_SDHC_fat32_3channel) Implement FAT32 file format of 1024 bytes size of each sector on s SDCARD Flash memory which is connected on SPI PORT of LPC2148. The interface signals are SCK (PORT0.17), MISO(PORT0.18), MOSI (PORT0.19), SSEL(PORT0.20). Use LCD 16x2 LCD, 8-bit Mode, RS=PORT0.2, RW=PORT0.3, E=PORT0.4, D0 to D7 = PORT1.16 to PORT1.23 to display the status of SDCARD read, write and error operations. Read ADC 3 Channels data and create a file LOG1.TXT and store the data in three columns along with DATE, TIME.

Project-2: (code: L&T/1. Blinky, 2. Serial_without_critical, 3_serial_with_semaphore, 4_soft_timers) Implement RTOS suing FREE RTOS functions. Demonstrate use of scheduler, semaphores, critical code, and soft timers.

- LED connected on (Port P0.31) of LPC2148 implement scheduler function of RTOS with a FLAH rate of 1000ms.

- Create a Multi tasking (two tasksS1, S2) environment using UART-0  where both use the scheduler.

- Create a Multi tasking (two tasksS1, S2) environment using UART-0 and implement MUTEX semaphore functions.

- Create a MUTEX for UART-0 and also create two software timers which will generate interrupts at 2000ms & 4000ms.

Project3-6: (10_Serial_FFT)(11_Test_GPS)(12_Test_GSM)(13_Test_GSM_GPS) Interface GSM, GPS of SIM 908 to LPC2148 UART-1 and Implement AT commands.

Project-7: (14_DC_STEP): Interface a Stepper Motor Driver (L293D) to LPC2148 PORTS 0.20 to PORT0.23 and write a program for bidirectional rotation. Connect +12V supply to DC and Stepper Motor Driver.

Example Programs:

- Blink the LED (Port P0.31):

#include <LPC214X.h>

void wait(int count)

{    int j=0,i=0;

for(j=0;j<count;j++)

{    /* At 60Mhz, the below loop introduces delay of 10 us */

for(i=0;i<35;i++);   }}

int main()

{ 	unsigned long i=0;

IO0DIR|=((unsigned long)1<<31);

while(1){

IO0SET=((unsigned long)1<<31);

wait(100000);

IO0CLR=((unsigned long)1<<31);

wait(100000);

i++; 	}}

- DAC waveform generation:

#include <LPC214x.H>                       /* LPC214x definitions */

#include <stdio.h>

#include <math.h>

void delay(int );

int main (void)

{

unsigned char wav[100];

unsigned char ucTemp,buf[16],j=0;

PINSEL1=(1<<19);

for(ucTemp=0;ucTemp<100;ucTemp++)

{

wav[ucTemp]=100*sin(((float)ucTemp/100)*3.1415);

wav[ucTemp]=(wav[ucTemp]*wav[ucTemp])/100;

}

while(j<1000)

{

for(ucTemp=0;ucTemp<100;ucTemp++)

{

DACR=wav[ucTemp]<<8;

}

j++;

}}

- Serial Communication:

#include <LPC214X.h>

#include "Serial.h"

void wait(int count)

{

int j=0,i=0;

for(j=0;j<count;j++)

{

/* At 60Mhz, the below loop introduces  delay of 10 us */

for(i=0;i<35;i++);

}

}

int main()

{

uart0_init();

IO0DIR|=((unsigned long)1<<31);

while(1){ uart0_puts("\n\rHello World!!!");

wait(1000);

} }

Serial.c:

#include <LPC214x.H>                     /* LPC21xx definitions               */

#include <stdio.h>

#include "Serial.h"

#define CR     0x0D

/* implementation of putchar (also used by printf function to output data)    */

int sendchar (int ch)  {                 /* Write character to Serial Port    */

if (ch == '\n')  {

while (!(U1LSR & 0x20));

U1THR = CR;                          /* output CR */

}

while (!(U1LSR & 0x20));

return (U1THR = ch);

}

int uart0_getkey (void)  {                     /* Read character from Serial Port   */

while (!(U0LSR & 0x01));

return (U0RBR);

}

int uart1_getkey (void)  {                     /* Read character from Serial Port   */

while (!(U1LSR & 0x01));

return (U1RBR);

}

void uart1_init()

{

/* initialize the serial interface

Baud= Pclk * mulVal

-----------------

16*(256*U0BLM+U0DLL)*(mulVal+divVal)

*/

PINSEL0 |= 0x00050000;           /* Enable RxD1 and TxD1                     */

U1LCR = 0x83;                   /* 8 bits, no Parity, 1 Stop bit            */

U1DLL = 71;                     /* 9600 Baud Rate @ 15MHz VPB Clock         */

U1FDR=0x00000083;					//MulVal=8 DivVal=3

U1LCR = 0x03;                   /* DLAB = 0  */

U1FCR=0x01;

}

//---------------- Function for send character 1 time via UART1-----------------------//

void uart1_putc(char c)

{

while(!(U1LSR & 0x20)); // Wait until UART1 ready to send character

U1THR = c;  // Send character

}

//-------------------------- Function for send string via UART1---------------------------------//

void uart1_puts(char *p)

{

while(*p) // Point to character

{

uart1_putc(*p++);   // Send character then point to next character

}

}

//---------------------------- Function for Initial UART0 ----------------------------------------//

void uart0_init()

{

/* initialize the serial interface

Baud= Pclk * mulVal

-----------------

16*(256*U0BLM+U0DLL)*(mulVal+divVal)

*/

PINSEL0 |= 0x00000005;           /* Enable RxD0 and TxD0                     */

U0LCR = 0x83;                   /* 8 bits, no Parity, 1 Stop bit            */

//U0DLL = 71;                     /* 9600 Baud Rate @ 15MHz VPB Clock         */

//U0FDR=0x00000083;				  /*MulVal=8 Divide Val=3						*/

U0DLL=97;

U0LCR = 0x03;                   /* DLAB = 0                                 */

}

//----------------------- Function for send character 1 time via UART0-----------------------//

void uart0_putc(char c)

{

while(!(U0LSR & 0x20)); // Wait until UART0 ready to send character

U0THR = c; // Send character

}

//------------------------- Function for send string via UART1---------------------------------//

void uart0_puts(char *p)

{

while(*p) // Point to character

{

uart0_putc(*p++);  // Send character then point to next character

}

}

void uart0_dump(char *p,unsigned short len,unsigned char mod)

{

unsigned char temp[3];

unsigned short i=0;

for(;i<len;i++){

if(i%mod==0)

uart0_puts("\n\r");

sprintf((char *)temp,"%02x ",p[i]);

uart0_puts((char *)temp);

}

}

Serial.h:

int uart0_getkey(void);

int uart1_getkey(void);

void uart1_init (void);

void uart0_init (void);

void uart1_putc	(char);

void uart0_putc	(char);

void uart1_puts	(char *);

void uart0_puts	(char *);

- Read ADC display on LCD:

#include <stdio.h>

#include <LPC214x.H>                       /* LPC214x definitions */

#include "lcd.h"

#include "adc.h"

/** Function Name : wait()    **/

void wait(int count)

{   int j=0,i=0;

for(j=0;j<count;j++)

{     /* At 60Mhz, the below loop introduces  delay of 10 us */

for(i=0;i<35;i++);   } }

/**	Function Name :	process_adc()    **/

void process_adc(void)

{ unsigned short adc_value = 0;

unsigned char buf[16] = {0};

adc_value = adc_read(ADC0, CHANNEL_1);

sprintf((char *)buf, "ADC:%d ", adc_value);

lcd_putstring(LINE1, (char *)buf); }

/**	Function Name :	main (void)   **/

int main (void)

{

init_adc0();						// Initialize ADC

init_lcd();						// Initialize LCD

lcd_clear();						// clear display

lcd_putstring(0,"LCD TEST 123");

lcd_putstring(1,"LPC2148");

wait(900000);

lcd_clear();						// clear display

while(1)

{     process_adc();			// Read ADC value and display it on first line of LCD

wait(30000);	   } }

LCD.c:

#include <LPC214x.H>                       /* LPC214x definitions */

#include "lcd.h"

#define LCD_BACK_LIGHT_TIMEOUT 1000

#define LCD_BACKLIGHT (1 << 27)

#define LCD_BACK_LIGHT_DIR IO1DIR

#define LCD_BACK_LIGHT_SET IO1SET

#define LCD_BACK_LIGHT_CLR IO1CLR

#define LCD_DATA_DIR	   IO1DIR

#define LCD_DATA_SET	   IO1SET

#define LCD_DATA_CLR	   IO1CLR

#define LCD_CTRL_DIR	   IO0DIR

#define LCD_CTRL_SET       IO0SET

#define LCD_CTRL_CLR       IO0CLR

#define LCDRS	           (1 << 2)

#define LCDRW	           (1 << 3)

#define LCDEN	           (1 << 4)

#define LCD_D4 (1 << 20)

#define LCD_D5 (1 << 21)

#define LCD_D6 (1 << 22)

#define LCD_D7 (1 << 23)

#define LCD_DATA_MASK           (LCD_D4 | LCD_D5 | LCD_D6 | LCD_D7)

#define LCD_BUSY_FLAG           LCD_D7

#define LCD_CONTROL_MASK        0x0000001C

/**  Function Name : delay()    **/

void delay(int count)

{

int j=0,i=0;

for(j=0;j<count;j++)

{     /* At 60Mhz, the below loop introduces     delay of 10 us */

for(i=0;i<35;i++);   } }

/**   Function Name :	wait_lcd()    **/

void wait_lcd( void )

{

LCD_CTRL_CLR |=  LCDRS;

LCD_CTRL_SET |=  LCDRW |LCDEN;

while(IO1PIN & LCD_BUSY_FLAG);	/* wait for busy flag to become low */

LCD_CTRL_CLR |= LCDEN | LCDRW;

LCD_DATA_DIR |= LCD_DATA_MASK;

delay(100);

}

/**   Function Name :	lcd_command_write()    **/

void lcd_command_write( unsigned char command )

{

unsigned char temp=0;

unsigned int temp1=0;

temp=command;

temp=(temp>>4)&0x0F;

temp1=(temp<<20)&LCD_DATA_MASK;

LCD_CTRL_CLR = LCDRS;

LCD_CTRL_SET = LCDEN;

LCD_DATA_CLR = LCD_DATA_MASK;

LCD_DATA_SET = temp1;

delay(10000);

LCD_CTRL_CLR = LCDEN;

temp=command;

temp&=0x0F;

temp1=(temp<<20)&LCD_DATA_MASK;

delay(100*2);

LCD_CTRL_CLR |= LCDRS;

LCD_CTRL_SET |= LCDEN;

LCD_DATA_CLR = LCD_DATA_MASK;

LCD_DATA_SET = temp1;

delay(10000);

LCD_CTRL_CLR |= LCDEN;

delay(10000);

//wait_lcd();

}

/**   Function Name :	set_lcd_port_output()   **/

void set_lcd_port_output( void )

{

LCD_CTRL_DIR |= ( LCDEN | LCDRS | LCDRW );

LCD_CTRL_CLR |= ( LCDEN | LCDRS | LCDRW );

LCD_DATA_DIR |= LCD_DATA_MASK;

}

/**  Function Name :	lcd_clear()   **/

void lcd_clear( void)

{

lcd_command_write( 0x01 );

}

/**   Function Name :	lcd_gotoxy()    **/

int lcd_gotoxy( unsigned int x, unsigned int y)

{

int retval = 0;

if( (x > 1) && (y > 15) )

{

retval = -1;

} else {

if( x == 0 )

{

lcd_command_write( 0x80 + y );	/* command - position cursor at 0x00 (0x80 + 0x00 ) */

} else if( x==1 ){

lcd_command_write( 0xC0 + y );	/* command - position cursor at 0x40 (0x80 + 0x00 ) */

}   }

return retval;}

/**  Function Name :	lcd_data_write()    **/

void lcd_data_write( unsigned char data )

{

unsigned char temp=0;

unsigned int temp1=0;

temp=data;

temp=(temp>>4)&0x0F;

temp1=(temp<<20)&LCD_DATA_MASK;

LCD_CTRL_SET |= LCDEN|LCDRS;

LCD_DATA_CLR = LCD_DATA_MASK;

LCD_DATA_SET = temp1;

delay(10000);

LCD_CTRL_CLR |= LCDEN;

temp=data;

temp&=0x0F;

temp1=(temp<<20)&LCD_DATA_MASK;

LCD_CTRL_SET |= LCDEN|LCDRS;

LCD_DATA_CLR = LCD_DATA_MASK;

LCD_DATA_SET = temp1;

delay(10000);

LCD_CTRL_CLR |= LCDEN;

//wait_lcd();

delay(1000);

}

/** Function Name :	lcd_putchar()   **/

void lcd_putchar( int c )

{

lcd_data_write( c );

}

/**   Function Name :	lcd_putstring()  **/

void lcd_putstring( unsigned char line, char *string )

{

unsigned char len = MAX_CHAR_IN_ONE_LINE;

lcd_gotoxy( line, 0 );

while(*string != '\0' && len--)

{

lcd_putchar( *string );

string++;

}

}

/**  Function Name :	lcd_backlight_on()   **/

void lcd_backlight_on()

{

LCD_BACK_LIGHT_DIR |= LCD_BACKLIGHT;

LCD_BACK_LIGHT_SET |= LCD_BACKLIGHT;

}

/**  Function Name : turn_off_lcd_back_light()  **/

void turn_off_lcd_back_light_cb(void)

{

LCD_BACK_LIGHT_DIR |= LCD_BACKLIGHT;

LCD_BACK_LIGHT_CLR |= LCD_BACKLIGHT;

}

/** Function Name :	init_lcd()   **/

void init_lcd( void )

{

set_lcd_port_output();

delay(100*100);

lcd_command_write(0x28);     /*   4-bit interface, two line, 5X7 dots.        */

lcd_clear() ;                /*   LCD clear                                   */

lcd_command_write(0x02);     /*   cursor home                                 */

lcd_command_write(0x06);     /*   cursor move direction                       */

lcd_command_write(0x0C) ;    /*   display on      */

lcd_gotoxy(0, 0);

lcd_clear();

}

ADC.c:

#include "adc.h"

#include <LPC214x.H>                       /* LPC214x definitions */

/*  Function Name :    init_adc0()

Description :	Initialises the ADC0   */

void init_adc0(void)

{

PINSEL1 = (PINSEL1 & ~(3 << 25)) | (1 << 24);

}

/**   Function Name :init_adc1()   **/

void init_adc1(void)

{

}

/** 	Function Name : adc_read()  **/

unsigned short adc_read(unsigned char adc_num, unsigned char ch)

{

unsigned int i=0;

switch(adc_num)

{

case ADC0:

AD0CR = 0x00200D00 | (1<<ch);    // select channel

AD0CR |= 0x01000000; // Start A/D Conversion

do

{

i = AD0GDR; // Read A/D Data Register

} while ((i & 0x80000000) == 0); // Wait for end of A/D Conversion

break;

case ADC1:

AD1CR = 0x00200D00 | (1<<ch); // select channel

AD1CR |= 0x01000000; // Start A/D Conversion

do

{

i = AD1GDR; // Read A/D Data Register

} while ((i & 0x80000000) == 0); // Wait for end of A/D Conversion

break;

}

return (i >> 6) & 0x03FF; // bit 6:15 is 10 bit AD value

}

ADC.h:

#ifndef _ADC_H

#define _ADC_H

#define END_0F_CONVERSION_BIT		(1<<31)

#define END_OF_CONVERSION(i)		(i & END_0F_CONVERSION_BIT)

#define ADC_VALUE_MASK                  0x03FF

#define ADC_CHANNEL_NUMBER_MASK		0x07

#define CHANNEL_0   0

#define CHANNEL_1   1

#define CHANNEL_2   2

#define CHANNEL_3   3

#define CHANNEL_4   4

#define CHANNEL_5   5

#define CHANNEL_6   6

#define CHANNEL_7   7

/* A/D Control Register */

#define AD0_0		0x00000001

#define AD0_1		0x00000002

#define AD0_2		0x00000004

#define AD0_3		0x00000008

#define AD0_4		0x00000010

#define AD0_5		0x00000020

#define AD0_6		0x00000040

#define AD0_7		0x00000080

#define AD1_0		0x00000001

#define AD1_1		0x00000002

#define AD1_2		0x00000004

#define AD1_3		0x00000008

#define AD1_4		0x00000010

#define AD1_5		0x00000020

#define AD1_6		0x00000040

#define AD1_7		0x00000080

#define CLKDIV_BIT0	(1<<8)

#define CLKDIV_BIT1	(1<<9)

#define CLKDIV_BIT2	(1<<10)

#define CLKDIV_BIT3	(1<<11)

#define CLKDIV_BIT4	(1<<12)

#define CLKDIV_BIT5	(1<<13)

#define CLKDIV_BIT6	(1<<14)

#define CLKDIV_BIT7	(1<<15)

#define BURST		(1<<16)		//to eneble burst mode

#define CLKS_BIT0	(1<<17)

#define CLKS_BIT1	(1<<18)

#define CLKS_BIT2	(1<<19)

#define PDN 		(1<<21)

#define START_BIT0 	(1<<24)

#define START_BIT1 	(1<<25)

#define START_BIT2 	(1<<26)

#define EDGE 		(1<<27)

/* A/D Global Data Register */

#define OVERRUN		(1L<<30)

#define DONE		(1L<<31)

/* A/D Interrupt Enable Register  */

#define ADINTEN0 	(1<<0)

#define ADINTEN1 	(1<<1)

#define ADINTEN2 	(1<<2)

#define ADINTEN3 	(1<<3)

#define ADINTEN4 	(1<<4)

#define ADINTEN5 	(1<<5)

#define ADINTEN6 	(1<<6)

#define ADINTEN7 	(1<<7)

#define ADGINTEN	(1<<8)

#define ADC0 0

#define ADC1 1

#define ADC0CHANNELS  8

#define ADC1CHANNELS  8

#define POWER_DOWN_ADC0()    AD0CR &= ~(PDN)

#define POWER_UP_ADC0()      AD0CR |= (PDN)

#define POWER_DOWN_ADC1()    AD1CR &= ~(PDN)

#define POWER_UP_ADC1()      AD1CR |= (PDN)

//TN_EVENT EVT_ADC0;

//TN_EVENT EVT_ADC1;

void adc0_isr(void);

void adc1_isr(void);

void init_adc0( void );

void init_adc1( void );

unsigned short adc_read(unsigned char adc_num, unsigned char ch);

#endif